Sinusoidal Steady State Filters

Experiment 1, 2, & 3

Justin Fortner

March 16th, 2018

Section 04 Wednesday 12:20-2:20 (Tuwin)

Introduction

The purpose of lab 4 was to allow students to investigate the frequency domain characteristics of low and high pass filters. Three circuits will be observed in this lab, low pass RC, high pass RC and an inverting low pass op amp. For each experiment we must use the oscilloscope with probes set to measure at 10x in order to decrease loading effects of the probe itself. Using the oscilloscope we must take enough measurements in order to construct the bode and phase plots.

Low Pass RC Filter

The circuit in Figure 1 must be replicated in order to continue on in the experiment.

Figure 1

Low Pass RC Filter Circuit Schematic for Lab 4, Experiment 1

In order to complete this circuit I used a 820 Ohm resistor and a 22 nF capacitor. In addition to the 820 Ohm resistor in the circuit, the wave generator has an internal resistance of 50 Ohms that must be taken into account when making calculations. The first calculation we must make is the corner frequency. The corner frequency is calculated using the equation fc=1/(2πRC). For this circuit the corner frequency is 8315.305 Hz. The phase shift at this frequency is calculated using the equation ϕ= -arctan(2πfRC) was found to be -44.99∘. Additional variations of the corner frequencies and their corresponding phase shifts can be seen in Figure 2 below.

| fc | f(Hz) | Vin(V) | Vout(V) | Phase (Expected) | Phase (Experiment) | % Difference |
| --- | --- | --- | --- | --- | --- | --- |
| 0fc | 0 | 0 | 0 | -0∘ | -0∘ | 0% |
| .001fc | 8.315305 | 4.16 | 4.16 | -.057∘ | -.055∘ | 3.6% |
| .01fc | 83.15305 | 4.3 | 4.3 | -.579∘ | -.513∘ | 11.4% |
| .1fc | 831.5305 | 4.3 | 4.2 | -5.71∘ | -5.45∘ | 4.55% |
| .5fc | 4157.652 | 4.1 | 3.7 | -26.56∘ | -24.74∘ | 6.85% |
| fc | 8315.305 | 4.1 | 3 | -44.99∘ | -44.01∘ | 2.1% |
| 5fc | 41576.52 | 3.9 | .45 | -78.69∘ | -79∘ | .39% |
| 10fc | 83153.05 | 3.9 | .4 | -84.29∘ | -84∘ | .34% |
| 25fc | 207882.6 | 3.9 | .19 | -87.709∘ | -88.2∘ | .55% |
| 50fc | 415765.2 | 3.9 | .09 | -88.85∘ | -88.78∘ | .07% |
| 100fc | 831530.5 | 3.7 | .05 | -89.42∘ | -89.8∘ | .42% |
| 1000fc | 8315305 | 3.7 | .02 | -89.94∘ | -90∘ | .06% |

Figure 2

Low Pass RC Filter Circuit Frequencies and Phase Shifts, Experiment 1

The resulting bode plot from the information in Figure 2 can be seen below in Figure 3.

Figure 3

Low Pass RC Filter Circuit Bode Plot, Experiment 1

The bode plot shows that as the frequency increases the gain will decrease from 0 into the negatives. The resulting phase plot from the information in Figure 2 can be seen below in Figure 4.

Figure 4

Low Pass RC Filter Circuit Phase Plot, Experiment 1

The phase plot shows that the phase angle at the corner frequency is -45∘. The phase angle is always negative and has the steepest slope between .1fc and 10fc. The phase angle starts at 0∘and approaches -90∘.

High Pass RC Filter

The circuit in Figure 5 must be replicated in order to continue on in the experiment.

Figure 5

Low Pass RC Filter Circuit Schematic for Lab 4, Experiment 2

In order to complete this circuit I used a 820 Ohm resistor and a 22 nF capacitor. In addition to the 820 Ohm resistor in the circuit, the wave generator has an internal resistance of 50 Ohms that must be taken into account when making calculations. The first calculation we must make is the corner frequency. The corner frequency is calculated using the equation fc=1/(2πRC). For this circuit the corner frequency is 8315.305 Hz. The phase shift at this frequency is calculated using the equation ϕ= -arctan(2πfRC) was found to be -44.99∘. Additional variations of the corner frequencies and their corresponding phase shifts can be seen in Figure 6 below.

| fc | f(Hz) | Vin(V) | Vout(V) | Phase (Expected) | Phase (Experiment) | % Difference |
| --- | --- | --- | --- | --- | --- | --- |
| 0fc | 0 | 0 | 0 | 0∘ | 0∘ | 0% |
| .001fc | 8.315305 | 4.3 | .003 | 89.94∘ | 90.77∘ | .91% |
| .01fc | 83.15305 | 4.3 | .05 | 89.42∘ | 89.77∘ | .39% |
| .1fc | 831.5305 | 4.3 | .4 | 84.29∘ | 85.36∘ | 1.24% |
| .5fc | 4157.652 | 4.1 | 1.73 | 63.53∘ | 63.75∘ | .34% |
| fc | 8315.305 | 4.1 | 2.77 | 45∘ | 45.95∘ | 2.06% |
| 5fc | 41576.52 | 4 | 2.79 | 11.3∘ | 11.99∘ | 5.75% |
| 10fc | 83153.05 | 4 | 2.81 | 5.71∘ | 5.65∘ | 1.16% |
| 25fc | 207882.6 | 4 | 3.78 | 2.29∘ | 2.41∘ | 4.97% |
| 50fc | 415765.2 | 4 | 4 | 1.14∘ | 1.18∘ | 3.38% |
| 100fc | 831530.5 | 4 | 4 | .572∘ | .61∘ | 6.2% |
| 1000fc | 8315305 | 4 | 4 | .057∘ | .06∘ | 5% |

Figure 6

Low Pass RC Filter Circuit Frequencies and Phase Shifts, Experiment 2

The resulting bode plot from the information in Figure 6 can be seen below in Figure 7.

Figure 7

Low Pass RC Filter Circuit Bode Plot, Experiment 2

The bode plot shows that as the frequency increases the gain will increase from the negatives towards 0. The resulting phase plot from the information in Figure 6 can be seen below in Figure 8.

Figure 8

Low Pass RC Filter Circuit Phase Plot, Experiment 2

The phase plot shows that the phase angle at the corner frequency is 45∘. The phase angle is always positive and has the steepest slope between .1fc and 10fc. The phase angle starts at 90∘and approaches 0∘.

Inverting Low Pass Op Amp Filter

The circuit in Figure 9 must be replicated in order to continue on in the experiment.

Figure 9

Inverting Low Pass Op Amp Filter Circuit Schematic for Lab 4, Experiment 3

In order to complete this circuit I used a 820 Ohm resistor and a 22 nF capacitor. In addition to the 820 Ohm resistor in the circuit, the wave generator has an internal resistance of 50 Ohms that must be taken into account when making calculations. A 1.6 KOhm resistor is used in parallel with a 22 nF capacitor. The first calculation we must make is the corner frequency. The corner frequency is calculated using the equation fc=1/(2πRC). For this circuit the corner frequency is 4521.447 Hz. The phase shift at this frequency is calculated using the equation ϕ= -arctan(f/fc) was found to be -45∘. However because this is an inverting circuit there is a 180∘phase shift. This making the phase shift at the corner frequency 135∘ Additional variations of the corner frequencies and their corresponding phase shifts can be seen in Figure 10 below.

| fc | f(Hz) | Vin(mV) | Vout(mV) | Phase (Expected) | Phase (Experiment) | % Difference |
| --- | --- | --- | --- | --- | --- | --- |
| 0fc | 0 | 0 | 0 | 0∘ | 0∘ | 0% |
| .001fc | 4.521447 | 201 | 450 | 179.94∘ | 180∘ | .03% |
| .01fc | 45.21447 | 201 | 450 | 179.42∘ | 180∘ | .32% |
| .1fc | 452.1447 | 201 | 450 | 174.29∘ | 173.98∘ | .17% |
| .5fc | 2260.723 | 201 | 375 | 153.43∘ | 151.67∘ | 1.14% |
| fc | 4521.447 | 201 | 310 | 135∘ | 133∘ | 1.48% |
| 5fc | 22607.23 | 201 | 50 | 101.30∘ | 99.79∘ | 1.49% |
| 10fc | 45214.47 | 201 | 45 | 95.71∘ | 94.31∘ | 1.46% |
| 25fc | 113036.1 | 201 | 22 | 92.29∘ | 92∘ | .31% |
| 50fc | 226072.3 | 201 | 12 | 91.14∘ | 90.97∘ | .18% |
| 100fc | 452144.7 | 201 | 5 | 90.57∘ | 90.22∘ | .38% |
| 1000fc | 4521447 | 201 | 2 | 90.05∘ | 90∘ | .05% |

Figure 9

Inverting Low Pass Op Amp Filter Circuit Frequencies and Phase Shifts, Experiment 3

The resulting bode plot from the information in Figure 9 can be seen below in Figure 10.

Figure 10

Inverting Low Pass Op Amp Filter Circuit Bode Plot, Experiment 3

The bode plot shows that as the frequency increases the gain will decrease from roughly 10  towards the negatives. The resulting phase plot from the information in Figure 9 can be seen below in Figure 11.

Figure 11

Inverting Low Pass Op Amp Filter Circuit Phase Plot, Experiment 3

The phase plot shows that the phase angle at the corner frequency is 135∘. The phase angle is always positive and has the steepest slope between .1fc and 10fc. The phase angle starts at 180∘and approaches 90∘.

Conclusion

I had a fairly successful lab in lab 4. I successfully observed characteristics of all three circuits through their respective bode and phase plots.

Experiment 1 focuses on a passive RC low pass filter. The bode plot shows that as the frequency increases the gain will decrease from 0 into the negatives. The phase plot shows that the phase angle at the corner frequency is -45∘. The phase angle is always negative and has the steepest slope between .1fc and 10fc. The phase angle starts at 0∘and approaches -90∘.

Experiment 2 focuses on a passive RC high pass filter. The bode plot shows that as the frequency increases the gain will increase from the negatives towards 0. The phase plot shows that the phase angle at the corner frequency is 45∘. The phase angle is always positive and has the steepest slope between .1fc and 10fc. The phase angle starts at 90∘and approaches 0∘.

Experiment 3 focuses on a inverted active RC Op Amp low pass filter. The bode plot shows that as the frequency increases the gain will decrease from roughly 10  towards the negatives. The phase plot shows that the phase angle at the corner frequency is 135∘. The phase angle is always positive and has the steepest slope between .1fc and 10fc. The phase angle starts at 180∘and approaches 90∘.

This was a great lab, I accomplished and learned a lot about bode and phase plots as well as decibel and decibel notation.